实验二 处理器调度 —— 实时调度算法 EDF 和 RMS

- 实验目的：深入理解处理器调度算法，了解硬实时概念，掌握周期性实时任务调度算法 EDF（Earliest Deadline First）和 RMS（Rate Monotonic Scheduling）的可调度条件，并能在可调度情况下给出具体调度结果。

- 实验内容：在 Linux 环境中采用用户级线程模拟实现 EDF 和 RMS 两种实时调度算法。给定一组实时任务，按照 EDF 算法和 RMS 算法分别判断是否可调度。在可调度的情况下，创建一组用户级线程，分别代表各个实时任务，并按算法确定的调度次序安排各个线程运行，运行时在终端上画出其 Gantt 图。为了避免图形绘制淡化算法，Gantt 图可用字符表示。

- 实验准备

  - EDF 算法和 RMS 算法的可调度条件及调度原则：EDF 为可抢先式调度算法，其调度条件为∑(Ti​Ci​​)≤1 ；RMS 算法为不可抢先调度算法，其调度条件为∑(Ti​Ci​​)≤n(exp(nln(2)​)−1) 。

  - 在 Linux 环境中创建用户级线程的函数：创建用户级线程的库函数为int pthread_create (pthread_t* THREAD, pthread_attr_t* ATTR, void*(*START_ROUTINE)(void*), void* ARG) 。其中第 1 个参数是 pthread_t 型的指针，用于保存线程 id；第 2 个参数是 pthread_attr_t 型的指针，用于说明要创建的线程的属性，NULL 表示使用缺省属性；第 3 个参数指明了线程的入口，是一个只有一个 (void*) 参数的函数；第 4 个参数是传给线程入口函数的参数。例如pthread_create(tid, NULL, func, arg) 。

- 实验设计：实时任务用 task 数据结构描述，设计 4 个函数：select_proc () 用于实现调度算法，被选中任务执行 proc ()，在没有可执行任务时执行 idle ()，主函数 main () 初始化相关数据，创建实时任务并对任务进行调度。为模拟调度算法，给每个线程设置一个等待锁，暂不运行的任务等待在相应的锁变量上。主线程按调度算法唤醒一个子线程，被选中线程执行一个时间单位，然后将控制权交给主线程判断是否需要重新调度。

- 参考代码

c

#include "math.h"

#include "sched.h"

#include "pthread.h"

#include"stdlib.h"

#include "semaphore.h"

#include "stdio.h"

typedef struct/*实时任务描述*/

{

char task_id;

int call_num;/*任务发生次数*/

int ci;/*Ci*/

int ti;/*Ti*/

int ci_left;

int ti_left;

int flag;/*任务是否活跃,0为否,2为是*/

int arg;/*参数*/

pthread_t th;/*任务对应线程*/

} task;

void proc(int*args);

void*idle();

int select_proc();

int task_num=0;

int idle_num=0;

int alg;/*所选算法,1为EDF,2为RMS*/

int curr_proc=-1;

int demo_time=100;/*演示时间*/

task * tasks;

pthread_mutex_t proc_wait[100];

pthread_mutex_t main_wait, idle_wait;

float sum=0;

pthread_t idle_proc;

int main(int argc,char**argv)

{

pthread_mutex_init(&main_wait,NULL);

pthread_mutex_lock(&main_wait);/*下次执行lock等待*/

pthread_mutex_init(&idle_wait, NULL);

pthread_mutex_lock(&idle_wait);/*下次执行lock等待*/

printf(" Please input number of real time tasks; \n");

scanf("%d",&task_num);

tasks=(task*)malloc(task_num*sizeof( task));

int i;

for(i=0;i<task_num;i++)

{

pthread_mutex_init( &proc_wait[i],NULL);

pthread_mutex_lock(&proc_wait[i]);

}

for(i=0;i<task_num;i++)

{

printf("Please input task id,followed by Ci and Ti: \n");

getchar();

scanf("%c,%d,%d",&tasks[i].task_id,&tasks[i].ci,&tasks[i].ti);

tasks[i].ci_left = tasks[i].ci;

tasks[i].ti_left=tasks[i].ti;

tasks[i].flag = 2;

tasks[i].arg = i;

tasks[i].call_num=1;

sum = sum+(float)tasks[i].ci/(float)tasks[i].ti;

}

printf("Please input algorithm,1 for EDF,2 for RMS:");

getchar();

scanf("%d",&alg);

printf("Please input demo time:");

scanf("%d",&demo_time);

double r=1;/*EDF算法*/

if(alg==2)

{ /*RMS算法*/

r=((double)task_num)*(exp(log(2)/(double)task_num) - 1);

printf("r is %lf\n",r);

}

if(sum>r)

{ /*不可调度*/

printf("(sum=%f>r=%lf),not schedulable!\n",sum,r);

exit(2);

}

pthread_create(&idle_proc,NULL,(void*)idle,NULL);/*创建闲逛线程*/

for(i=0;i<task_num;i++)/*创建实时任务线程*/

pthread_create(&tasks[i].th,NULL,(void*)proc,&tasks[i].arg);

for(i=0;i<demo_time;i++)

{

int j;

if((curr_proc=select_proc(alg))!=-1)

{ /*按调度算法选线程*/

pthread_mutex_unlock(&proc_wait[curr_proc]);/*唤醒*/

pthread_mutex_lock(&main_wait);/*主线程等待*/

}

else

{ /*无可运行任务,选择闲逛线程*/

pthread_mutex_unlock(&idle_wait);

pthread_mutex_lock(&main_wait);

}

for(j=0;j<task_num;j++) /*Ti--,为0时开始下一周期*/

{

if(--tasks[j].ti_left==0)

{

tasks[j].ti_left = tasks[j].ti;

tasks[j].ci_left = tasks[j].ci;

pthread_create(&tasks[j].th,NULL,(void*)proc,&tasks[j].arg);

tasks[j].flag = 2;

}

}

printf("\n");

sleep(1);

}

}

void proc(int*args)

{

while(tasks[*args].ci_left>0)

{

pthread_mutex_lock(&proc_wait[*args]);/*等待被调度*/

if(idle_num!=0)

{

printf("idle(%d)",idle_num);

idle_num = 0;

}

printf(" %c%d",tasks[*args].task_id,tasks[*args].call_num);

tasks[*args].ci_left--;/*执行一个时间单位*/

if(tasks[*args].ci_left==0)

printf("(%d)",tasks[*args].ci);

tasks[*args].flag = 0;

tasks[*args].call_num ++;

pthread_mutex_unlock(&main_wait);/*唤醒主线程*/

}

}

void*idle()

{

while(1)

{

pthread_mutex_lock(&idle_wait);/*等待被调度*/

printf("->");/*空耗一个时间单位*/

idle_num++;

pthread_mutex_unlock(&main_wait);/*唤醒主控线程*/

}

}

int select_proc(int alg)

{

int j;

int temp1,temp2;

temp1=10000;

temp2=-1;

if((alg==2)&&(curr_proc!=-1)&&(tasks[curr_proc].flag!=0))

return curr_proc;

for(j=0;j<task_num;j++)

{

if(tasks[j].flag == 2)

{

switch(alg)

{

case 1:/*EDF算法*/

if(temp1>tasks[j].ti_left)

{

temp1 = tasks[j].ti_left;

temp2=j;

}

break;

case2:/*RMS算法*/

if(temp1>tasks[j].ti)

{

temp1

你的2.62.7呢

2.6 实验结果

plaintext

# 编译

gcc -lpthread -lm test_scheduler.c -o scheduler.out

# 算法EDF结果

[root@localhost test]#./scheduler.out

Please input number of real time tasks: 2

Please input task id, followed by Ci and Ti: a,10,20

Please input task id, followed by Ci and Ti: b,25,50

Please input algorithm, 1 for EDF,2 for RMS:1

Please input demo time:200

alalalalalalalalalal(10)b1blblblblblblblblbla2a2a2a2a2a2a2a2a2a2(10)blblblblblblblb1 blblblblb1blb1(25)a3a3a3a3a3a3a3a3a3a3(10)b2b2b2b2b2a4a4a4a4a4a4a4a4a4a4(10)b2b2b2b 2b2b2b2b2b2b2a5a5a5a5a5a5a5a5a5a5(10)b2b2b2b2b2b2b2b2b2b2(25)a6a6a6a6a6a6a6a6aba6(10) b3b3b3b3b3b3b3b3b3b3a7a7a7a7a7a7a7a7a7a7(10)b3b3b3b3b3b3b3b3b3b3b3b3b3b3b3(25)a88a 8a8a8a8a8a8a8a8a(10)b4b4b4b4b4a9a9a9a9a9a9a9a9a9a9(10)b4b4b4b4b4b4b4b4b4b4a10a10a10a 10a10a10a10a10a10a10(10)b4b4b4b4b4b4b4b4b4b4(25)

# 算法RMS结果

[root@localhost test]#./scheduler.out

Please input number of real time tasks: 3

Please input task id, followed by Ci and Ti:

a,20,100,

Please input task id, followed by Ci and Ti:

b,40,150,

Please input task id, followed by Ci and Ti:

c,100,350,

Please input algorithm, 1 for EDF,2 for RMS:2

Please input demo time:400

r is 0.779763

alalalalalalalalalalalalalalalalalalalal(20)b1blb1b1blb1b1b1b1b1b1b1b1b1b1b1b1b1b1b1

blb1b1b1b1b1b1b1b1b1b1b1b1b1b1b1b1b1b1(40)clelclclclclclclclclelclclclclclclclclcl clclclelelclclelelclclelelclclelelclclclelelclelelelelclclclelelclclclclclclelclclclclclclclel(100)a2a2a2a2a 2a2a2a2a2a2a2a2a2a2a2a2a2a2a2a2(20)b2b2b2b2b2b2b2b2b2b2b2b2b2b2b2b2b2b2b2b2b2b2b2b2b 2b2b2b2b2b2b2b2b2b2b2b2b2b2b2b2(40)a3a3a3a3a3a3a3a3a3a3a3a3a3a3a3a3